A BRIEF SUMMARY AND INTERPRETATION

OF KEY POINTS, FACTS AND CONCLUSIONS

FOR

UNIVERSITY OF WISCONSIN STUDY:

“LIFE CYCLE COMPARISON OF FIVE ENGINEERED

SYSTEMS FOR MANAGING FOOD WASTE”

By

William.F.Strutz

Staff Engineer

IN-SINK-ERATOR

APRIL 1998

In order to develop a factual database, relative to the actual merits and concerns for different systems of managing food waste, the National Association of Heating-Plumbing-Cooling Contractors commissioned a University of Wisconsin - Madison, Life Cycle Comparison of five engineered Systems for Managing Food Waste.

The comparison included the required land, total system energy, total system materials, total emissions to the environment and total system costs for each method.

Dr.Robert Ham of the Civil Engineering Department and one of the country’s recognised land-fill experts was chosen to lead and oversee the study.  Carol Diggelman, a graduate student in the Civil and Environmental Engineering Department at UW and a Professor in Environment Engineering at the Milwaukee School of Engineering in Milwaukee, Wisconsin, was chosen to do the research.

The results of this four year research project are contained in a 571 page report titled “Life-Cycle Comparison of Five Engineered Systems for Managing Food Waste”, which compares all five systems on the basis of processing 100 kilograms of food waste.

The five systems are;

- Food Waste Disposer plus a Publicly Owned Treatment Works (FWD/POTW)

- Municipal Solid Waste Collection/Land-filling (MSW Collection/Land-filling)

- Municipal Solid Waste Collection/Composting (MSW Collection/Land-filling)

- Municipal Solid Waste Collection/Waste to Energy (MSW Collection/WTE; Incineration)

- Food Waste Disposer plus an On-Site (Septic) System (FWD/OSS)

The first four systems are based on specific state of the art operational systems.  The on-site system design is based on simply increasing the septic tank and drain field size by 25% to accommodate a food waste disposer.  This requirement is based on a typical required increase of 25% solids loading to the system, based on previous research.

Assumptions for the study based on the best available data:

- The base of 100 kilograms of food waste was chosen as a convenient basis of comparison for the five engineered systems.  An average person generates 0.29 pounds of food waste per day.  Of this, 75% or 0.21 pounds per day is processed through a food waste disposer.  100 kilograms of food waste is therefore the amount processed by the “average” U.S family of 2.63 persons over a period of 382 days, or just slightly over 1 year.

- Typical food waste is 70% water and 30% solids.

- The typical composition of food waste and human waste solids is:

|  | % C | % H | %0 | %N | %S |
| --- | --- | --- | --- | --- | --- |
| Human Waste, Solid Organic | 59.7 | 9.5 | 23.8 | 7.0 | 0 |
| C10 H19 O3 N |  |  |  |  |  |
| Food Waste, Solid Organic | 50.5 | 6.72 | 39.6 | 2.74 | 0.44 |
| C21.53 H34.21O12.66 N1.00 S0.07 |  |  |  |  |  |

- The final destination of food waste in the U.S;

- Municipal Solid Waste Collection/Land-Fill………………………………..…41%

- Food Waste Disposer/Publicly Owned Treatment Works……………………..37%*

- Food Waste Disposer/On-Site (Septic) System………………………………..12%*

- Municipal Solid Waste Collection/Waste To Energy (Incineration)…………..10%

- Municipal Solid Waste Collection/Composting…………………………………0%

*Wastewater food waste includes contributions from dishwashers and kitchen sinks.

CONCLUSION

Of the five alternative food waste systems measured, a food waste disposer processing food waste through a publicly owned treatment works has the lowest cost to the municipality; the least air emissions especially greenhouse gases; converts the food WASTE to a RESOURCE which may be recycled; and as a result overall is the most environmentally friendly and sustainable option for recycling non-edible food RESOURCES.

The food waste disposer is also the most convenient method of disposing food waste and is most likely to be used as the vehicle for source separation of food waste from the solid waste stream.

LIFE_CYCLE COSTS

In terms of life cycle costs, the systems ranked in this order (lowest to highest):

- Municipal Solid Waste Collection / Land-filling

- Municipal Solid Waste Collection / Composting

- Food Waste Disposer / Publicly Owned Treatment Works

- Municipal Solid Waste Collection / Waste to Energy (Incineration)

- Food Waste Disposer / On-Site (Septic) Tank

However, in terms of direct costs to the municipality, the Food Waste Disposer/Publicly Owned Treatment Works, combination is, by far, the lowest cost.  The rankings and costs are;

- Food Waste Disposer / Publicly Owned Treatment Works……………….$0.49

- Municipal Solid Waste Collection / Land-filling…………………………..$13.65

- Municipal Solid Waste Collection / Composting………………………….$16.60

- Municipal Solid Waste Collection / Waste to Energy (Incineration)……..$20.30

- The Food Waste Disposer / On-Site Septic Tank is the highest cost at $67.20, but since all costs are borne directly by the homeowner, there is zero cost to the municipality.

Other benefits of the Food Waste Disposer / Publicly Owned Treatment works are;

- ENVIRONMENTAL

Environmentally, the disposer is the most convenient and most likely to be used method to achieve source separation of the putrescible waste from the solid waste stream.  Typically, 75% of non-edible food waste may be processed through a food waste disposer.  Presently 37% of the U.S household food waste goes to a POTW.  Food waste is typically 70% water, therefore utilising a wastewater treatment plant is a more natural method of processing this material than a method which collects this water and HAULS it to a “solid waste” facility.  Removing the putrescible food waste at the kitchen sink and diverting it from the solid waste stream also reduces disease causing vectors such as flies, rodents etc., that are attracted to food waste.

- RECYCLE FOOD NUTRIENTS

Since human waste influent to wastewater treatment systems is carbon limited (food carbon is exhaled by humans as carbon dioxide, enriching the sewage in Nitrogen and Phosphorus), the addition of food waste provides additional carbon to enhance the generation of biosolids.  The greater the amount of biosolids produced at the POTW, the greater the amount of nutrients, nitrogen and phosphorus, that is assimilated into the bio-mass, which is removed from the system as sludge and removed from the effluent.  When biosolids from the POTW, or septage processed through a POTW is applied to the soil, this is a variable method of recycling.

This process is the most beneficial for retaining the food waste nutrients in a form that can be recycled.

- HELPS LAND- FILLS

Land-filling is the method of disposing of solid waste that is required for every community in the U.S.  Presently 41% of U.S food waste goes to land-fills.  Hauling food waste that is 70% water to a “solid waste” facility represents over 72% of the life cycle costs of disposing of a potentially recyclable resource.  Adding this water to a well designed land fill also increases the quantity of leachate that is generated.

Due to the generally acidic nature of leachate from food waste, more metals are contained in the leachate than if the food waste was not in the land-fill.  This leachate is then typically hauled to a POTW for treatment to prevent the leachate from contaminating soils and potentially the groundwater (hauling the water not once, but twice).

Almost all of the nutrient value of the food is lost in the land-fill; the only portion that is potentially recycled is that which is captured in the leachate and processed through the POTW.  Eventually almost all of the carbon in the food waste at the land-fill is converted to methane.  In a well designed land-fill about 66% of the methane is recovered and beneficially reused as fuel.  However, the balance of 34% of the methane escapes to the atmosphere.  The methane gas has up to 25 times the global warming impact of carbon dioxide.

Putrescible food waste added to the normal household solid waste also adds to disease causing vector problems such as flies, rodents and roaches while awaiting collection.  In cities that have mandated food waste disposers, solid waste collection frequency has been reduced from twice weekly to weekly or even bi-weekly.

- BENEFITS OVER COMPOSTING

Municipal compost facilities are not as prevalent as land-fills.  They are considered an additional system and a land-fill is still required.  Hauling food waste that is 70% water to a compost facility represents over 59% of the life cycle costs of the compost operation.

Since municipal composting requires more moisture than is available in most materials, the addition of food waste does enhance the composting process.  This higher moisture content does require periodic turning of the material to keep the process aerobic.  If the process goes anaerobic, then there is the potential for significant odours to be generated and the result is community opposition to composting.

A number of facilities in the U.S have been shut down for odour problems.  This typically requires locating the facility away from population centres and hauling the high water content food waste longer distances.

Composting also results in the loss of most of the nutrients in the food to the extent that the resulting product is of very low value and typically is not worth the cost of hauling and spreading it onto soil.  In some communities the compost is of such low value that it is used as land-fill cover.  Food waste can be processed through the POTW at a much lower cost to the municipality, retain the nutrients for recycling, and reduce the atmospheric emissions.

- INDIVIDUAL COMPOST SYSTEMS LIMITATIONS

The typical backyard compost system, which is not analysed in this study is typically not as well maintained as a municipal system.  This results in more anaerobic conditions, more odours, more methane generated and released to the atmosphere, more potential for leachate to seep into groundwater and a low nutrient, low quality product.  These systems also tend to attract disease causing vectors.  Yet, many homeowners perceive composting to be the “ideal” method of recycling food waste.  The attached fact sheet based on information from the study contradicts this belief.

- WASTE TO ENERGY LIMITATIONS

Hauling food waste that is 70% water to a waste-to-energy facility represents over 48% of the life cycle costs of the operation.  Energy required for evaporation of the water in the food waste results in a very small net energy gain from the incineration of food waste.  Instead of the nutrients being captured for recycling, most are given off to the atmosphere as acidic or greenhouse gases.  Scrubbers are required in a well designed system to reduce these emissions and are a significant factor in making this the highest cost municipal system.  Refer to the attached fact sheet for a comparison of the emissions generated by the various methods.

- ON-SITE SYSTEM

An On-Site (Septic) System (OSS), is a requirement for processing wastewater in rural areas which are located beyond the municipal collection systems.  As stated earlier, the system for this study was based on a 25% larger system when a food waste disposer is used.  Since this is not a state of the art system, this results in the system with the highest life cycle cost.

However, systems with a disposer, an adequate soil type and a “standard” sized system have functioned trouble free for more than ten years in cold climates.  State of the art for on-site systems is the use of bio-additives to neutralise any potential additional loading due to food waste.

This allows using a “standard” sized system without any additional system cost, significantly reducing the life cycle cost of the FWD / OSS system.  A state of the art system such as IN-SINK-ERATOR’S Septic Disposer using Bio-Charge TM would reduce the size required for the system and reduce the system cost.

| ANAYLSIS OF FIVE FOOD WASTE MANAGEMENT SYSTEMS |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| FOR 100 Kilograms (220.5 pounds) of FOOD WASTE |  |  |  |  |  |
|  | DISPOSAL OPTION |  |  |  |  |
|  | FWD / POTW | MSW / L-F | MSW / COM | MSW /WTE | FWD / OSS |
| PARAMETER |  |  |  |  |  |
| Land Required (Sq. Ft) | 0.003 | 0.202 | 0.814 | 0.020 | 20.432 |
| Rank | 1 | 3 | 4 | 2 | 5 |
| Energy Required (Btu) | 45,744 | 80,112 | 143,299 | 286,433 | 925,824 |
| total-exportable food waste energy |  |  |  |  |  |
| Rank | 1 | 2 | 3 | 4 | 5 |
| Materials Required (pounds) | 287.4 | 338.2 | 89.6 | 116.1 | 488.1 |
| Rank | 3 | 4 | 1 | 2 | 5 |
| Oxygen Required (pounds) | 25 | 0 | 67 | 95 | 0 |
| Rank | 3 | 1 | 4 | 5 | 1 |
| Life Cycle Emissions (pounds) C2O | 97 | 81 | 100 | 140 | 130 |
| Rank | 2 | 1 | 3 | 5 | 4 |
| Methane | 0.00028 | 5 | 0.00028 | 0.00037 | 15 |
| Rank | 1 | 4 | 1 | 3 | 5 |
| Total Greenhouse Gas (C2O+4*methane) | 97 | 101 | 100 | 140 | 190 |
| Rank | 1 | 3 | 2 | 4 | 5 |
| Acid Gas (pounds) Nitrogen & Sulphur Oxides | 0.1 | <0.05 | 0.2 | 2.9 | 1.0 |
| Rank | 2 | 1 | 3 | 5 | 4 |
| Water Vapour (pounds) | 24 | 24 | 160 | 200 | 0 |
| Rank | 3 | 3 | 4 | 5 | 1 |
| Total Air Emissions | 120 | 110 | 260 | 343 | 140 |
| Rank | 2 | 1 | 4 | 5 | 3 |
| Total Water Required (pounds) | 2547 | 83 | 64 | 75 | 3994 |
| Carrier Water (pounds) | 2273 | 0 | 0 | 0 | 2273 |
| Rank | 4 | 3 | 1 | 2 | 5 |
| Water & Waterborne Waste | 2800 | 370 | 370 | 420 | 4800 |
| Rank | 3 | 1 | 1 | 2 | 5 |
| Solid Waste | 4.4 | 6 | 2.7 | 1.3 | 480 |
| Rank | 4 | 3 | 2 | 1 | 5 |
| Other | (sludge) 340 | (residue) 25 | (compost) 39 | (ash) 3.3 | (septage)310 |
| Rank | 5 | 2 | 3 | 1 | 4 |
| Life Cycle Costs |  |  |  |  |  |
| Disposer (homeowner cost or separation & convenience) |  |  |  |  |  |
| LOW | $8.83 | 0 | 0 | 0 | $8.83 |
| HIGH | $17.45 |  |  |  | $17.45 |
| Total System Cost |  |  |  |  |  |
| LOW | $9.32 | $13.65 | $16.60 | $20.30 | $58.58 |
| Rank | 1 | 2 | 3 | 4 | 5 |
| HIGH | $17.94 | $13.65 | $16.60 | $20.30 | $67.20 |
| Rank | 3 | 1 | 2 | 4 | 5 |
| Public Municipality Cost (external to home) | $0.49 | $13.65 | $16.60 | $20.30 | $67.20 |
| Rank | 1 | 2 | 3 | 4 | 5 |

KEY

- FWD						Food Waste Disposer

- POTW						Publicly Owned Treatment Works

- MSW						Municipal Solid Waste Collection

- MSW/COMPOST			Municipal Solid Waste Collection plus Composting

- WTE						Waste to Energy (Incineration)

- OSS							On-Site Septic System

The brief one page fact sheet titled “Life Cycle Analysis of Five Food Waste Management Systems” comparing the five systems for land, energy, materials, emissions and costs has been inserted above.  This fact sheet was developed to present the key parameters for each system and also ranks the systems for each parameter.

Carol Diggelman’s conclusions, recommendations, and two pages of detailed comparison charts from the original report are copied verbatim and attached for reference.  Copies of the 118 page Executive Summary and the 571 page Full Report are available upon request.

- As shown on page 11, in general, as total flows to the environment increase, so do total system costs, all per 100 kg food waste.  Rank by total system cost is a reasonable predictor of overall rank for the 12 selected parameters-total land, total system materials (minus food waste and carrier water), total system energy (minus food and carrier water energy), water, total system cost, air emissions, acid gases (Nox & Sox), greenhouse gases, wastewater, waterborne wastes, solid waste and food waste by-products.

- Total flows to the environment from wastewater systems are about 10 times those from MSW systems, primarily because of FWD carrier water.

- The FWD/OSS, the only rural system, ranked either first or second for most parameters.  Because a larger fraction of the total FWD/OSS was attributable to the 100kg of food waste; land, materials, energy and flows to the environment attributable to the 100kg were higher for the rural system than for the four municipal systems.

- The FWD/OSS has the highest flows to the environment of the five systems; most is water and waterborne wastes discharged with minimal performance control to the sub-surface.  About half of the effluent BOD5 is discharged directly to the absorption bed which may contribute to bio-mass assimilation and clogging in the absorption bed.  Although food waste carbon removes some ammonia-nitrogen from wastewater as it is assimilated into bio-mass, a system stoichiometric excess of ammonia-nitrogen remains to be discharged into the sub-surface, potentially by-passing plant root zones to pollute groundwater.

- The MSW Collection/WTE ranks second highest overall and for total system cost.  Burning food waste yields little exportable energy in these systems so diverting food waste to FWD/POTW systems should be defined as recycling and encouraged, just as diverting other recyclable with no heating value, such as metal and glass is encouraged.

- The FWD/POTW system ranks in the middle of the five systems overall and for total system materials and total system cost.  Most of the cost is for the FWD and is borne by the homeowner; the cost to process food waste through the POTW is less than $0.50 per 100kg of food waste.  The FWD/POTW has the lowest land and total system energy requirements, but the highest food waste by-product, sludge, requiring management.

- Wastewater collection and treatment systems and MSW collection systems and land-fills are required systems for both urban and rural residences for reasons of basic public health and sanitation.  When a FWD is incorporated in a household wastewater collection system, there is redundancy in food waste management and most food waste can be managed through either system.  Food waste going into a FWD/POTW system, from which either effluent and/or sludge is/are returned to agricultural soils in compliance with Federal and State regulations and in which methane is collected and combusted to produce electricity, is being effectively recycled.

- Adding food waste carbon to a carbon limited wastewater system contributes to a net removal of nutrients (nitrogen and phosphorus) from effluent, as nutrients are assimilated with carbon into bio-mass and removed from the system as sludge.

- Land requirements for each system give a first approximation of a system’s appropriation of and reduction in net primary productivity (mass of bio-mass produced per area or per joule of incident energy).  Even though impact to net primary productivity are beyond the scope of this project, the FWD/POTW system with the lowest land requirements has the lowest impact on net primary productivity from 100kg of food waste.  When coupled with potential increases in net primary productivity from effluent and sludge nutrients, this system is potentially the most sustainable of the five systems.

- The MW Collection/Compost system ranks lowest overall; it has the lowest total system materials and water requirements and generates the lowest amount of wastewater and waterborne wastes.  Food nutrients are returned to soil from compost systems.

- Composting is an optional food waste management system that increases redundancy in food waste management, however, wastewater collection/treatment and MSW Collection/Land-fill systems are still required.

- The MSW Collection/Land-fill system is the default system for food waste management.  It ranks next to lowest overall and lowest for cost.  It also ranks low for water, wastewater, total air emissions and food waste by-products.

- As indicated on page 12, for MSW systems the MSW Collection system contributes from half to ¾ of the total system cost.  Systematic diversion of wet, putrescible food waste from MSW to FWD’s has the potential to produce drier, more storable MSW and reduce the need for weekly collection and the cost of MSW Collection.

- The MSW Collection system requires about 17 times the land, about 18% of the total materials, 88% of the total system energy, is about half the high estimate and is about the same as the low estimate of the cost of the FWD; the total flows to the environment for the MSW Collection system are about 18% those of the FWD, because there is no carrier water.

- If household plumbing were redesigned to use non-potable water for flushing wastes (both human through toilets and food through FWD’s), diverting food wastes to municipal wastewater systems becomes a more sustainable choice.

FINAL RECOMMENDATIONS

- Diverting food waste through FWD’s to a POTW should be encouraged when solids’ handling systems are adequate, methane is combusted to generate energy, and effluent and/or sludge are returned to soil; food waste is effectively being recycled and should be so designated in Federal and State regulations.

- Benefits to MSW management systems from the systematic use of FWD’s should be quantified; because by transferring putrescible food waste from solid to wastewater management systems, there a reduction in regulatory requirements for MSW collection systems (weekly collection), land-fill systems (daily cover requirement), compost systems (more stringent management requirements) and reduced solids’ handling for WTE systems.

- Separate regulations that give different design requirements for POTW’s depending on FWD usage should be challenged, especially if no other household appliance or device is listed.

- To make the life cycle inventory a cost effective process, there needs to be an accurate, up-to-date data base of unit factors for water and waterborne wastes, air emissions, and solid waste for materials and fuels that is readily available to the public.

| Comparison of Land, Materials, Energy and Costs of Five System Used to Manage Food Waste |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | FWD+ |  | POTW+ | MSW |  | Compost + |  | WTE+ |  | Land-fill+ |
|  | FWD | OSS | OSS | POTW | FWD | Collection | Compost | collection | WTE | Collection | Land-fill | collection |
|  | table 4.17 | table 5.19 |  | table 6.103 |  | table 7.18 | table 8.17 |  | table 9.27 |  | table 7.43 |  |
| Land sq. ft | /100kg | /100kg | /100kg | /100kg | /100kg | /100kg | /100kg | /100kg | /100kg | /100kg | /100kg | /100kg |
|  | 0.0006 | 20.43 | 20.43 | 0.003 | 0.003 | 0.01 | 0.80 | 0.81 | 0.01 | 0.02 | 0.19 | 0.20 |
| Materials lb. | /100kg | /100kg | /100kg | /100kg | /100kg | /100kg | /100kg | /100kg | /100kg | /100kg | /100kg | /100kg |
| Construction & Land-fill materials | 0.1 | 3143.2 | 3143.3 | 7.9 | 8.0 | 2.7 | 5.9 | 8.6 | 5.0 | 7.7 | 243.7 | 246.4 |
| Equipment & Vehicles | 0.1 | neg. | 0.1 | 0.1 | 0.1 | 0.2 | 0.4 | 0.5 | 0.1 | 0.3 | 0.1 | 0.3 |
| Electricity | 1.4 | neg. | 1.4 | 1.4 | 2.8 | 5.4 | 9.5 | 14.9 | 22.1 | 27.4 | 0.0 | 5.4 |
| Natural Gas | 0.5 | neg. | 0.5 | 0.0 | 0.5 | N/I | 0.0 | 0.0 | 2.4 | 2.4 | 0.6 | 0.6 |
| Diesel Fuel | 0.1 | 12.9 | 13 | 0.1 | 0.2 | 1.4 | 0.2 | 1.6 | 1.9 | 3.3 | 1.4 | 2.8 |
| Gasoline | 0.7 | neg. | 0.7 | 0.0 | 0.7 | N/I | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| FWD Materials | 1.5 | 0 | 1.5 | 0.0 | 1.5 | N/A | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Water | 260.4 | 3733.4 | 3993.8 | 2286.3 | 2546.7 | 38.5 | 25.5 | 64 | 36.5 | 75 | 44.3 | 82.8 |
| Food Waste | 0.0 | 220.5 | 220.5 | 220.5 | 220.5 | 0.0 | 220.5 | 220.5 | 220.5 | 220.5 | 220.5 | 220.5 |
| Total | 264.9 | 7109.9 | 7374.8 | 2516.2 | 2781.1 | 48.2 | 261.9 | 310.1 | 288.4 | 336.6 | 510.5 | 558.7 |
| Total FW-CW | 264.9 | 4616.2 | 4881.1 | 22.5 | 287.4 | 48.2 | 41.4 | 89.6 | 67.9 | 116.1 | 290.0 | 338.2 |
| Energy Btu | /100kg | /100kg | /100kg | /100kg | /100kg | /100kg | /100kg | /100kg | /100kg | /100kg | /100kg | /100kg |
| Embodied Materials | 308 | 526506 | 526814 | 5707 | 6014 | 18983 | 13351 | 32334 | 2289 | 21272 | 6628 | 25611 |
| Embodied-Process equip/vehicles | 1477 | neg. | 1477 | 1021 | 2498 | 2027 | 6975 | 9002 | 2068 | 4095 | 1635 | 3662 |
| Electricity | 6177 | neg. | 6177 | 6056 | 12233 | 23373 | 41061 | 64434 | 99225 | 122598 | N/I | 23373 |
| Natural Gas | 13126 | neg. | 13126 | 418 | 13542 | N/I | N/I | N/I | 61347 | 61347 | 15299 | 15299 |
| Diesel | 3717 | 302149 | 305866 | 1659 | 5376 | 33856 | 3549 | 37405 | 43108 | 76963 | 31877 | 65733 |
| Gasoline | 16780 | N/I | 16780 | 52 | 16832 | N/I | N/I | N/I | N/I | N/I | N/I | N/I |
| FWD Material | 47197 | 0.0 | 47197 | 0.0 | 47197 | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| Water | 547 | 7840 | 8387 | 4798 | 5345 | 81 | 43 | 124 | 77 | 158 | 93 | 174 |
| Total | 89329 | 836495 | 925824 | 19708 | 109037 | 78320 | 64979 | 143299 | 208113 | 286433 | 55531 | 133851 |
| Total - Exportable FW Energy* | 89329 | 836495 | 925824 | -43585 | 45744 | 78320 | 64979 | 143299 | 208113 | 286433 | 1792 | 80112 |
| Costs - $ | $17.45 | $49.75 | $67.20 | $0.49 | $17.94 | $9.90 | $6.70 | $16.60 | $10.39 | $20.30 | $3.75 | $13.65 |
| Exportable Electricity* kWh | 0 | 0 | 0 | 19 | 19 | 0 | 0 | 0 | 0 | 0 | 16 | 16 |
| * Exportable Energy for POTW = 63,293 Btu/100kg FW; for Land-fill = 53,739 Btu/100kg FW |  |  |  |  |  |  |  |  |  |  |  |  |
| neg. - negligible |  |  |  |  |  |  |  |  |  |  |  |  |
| N/I - no information |  |  |  |  |  |  |  |  |  |  |  |  |
| N/A - Not Applicable |  |  |  |  |  |  |  |  |  |  |  |  |

| Summary of Life Cycle Emissions from Acquisition, Use and Decommissioning of Five Enhanced Systems for the Management of Food Waste |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | FWD+ |  | POTW + | MSW |  | Compost + |  | WTE + |  | Land-fill + |  |  |  |  |  |  |  |  |  |  |  |  |
|  | FWD | OSS | OSS | POTW | FWD | Collection | Compost | Collection | W-T-E | Collection | Land-fill | Collection |  |  |  |  |  |  |  |  |  |  |  |  |
| Air Emissions | lb/100kg | lb/100kg | lb/100kg | lb/100kg | lb/100kg | lb/100kg | lb/100kg | lb/100kg | lb/100kg | lb/100kg | lb/100kg | lb/100kg |  |  |  |  |  |  |  |  |  |  |  |  |
| Particulates | 2.8e-02 | 2.4e-01 | 2.7e-01 | 1.8e-03 | 3.0e-02 | 1.6e-02 | 1.4e-02 | 3.0e-02 | 9.4e-03 | 2.5e-02 | -1.2e-02 | 3.8e-03 |  |  |  |  |  |  |  |  |  |  |  |  |
| Nitrogen Oxides | 4.5e-02 | 6.2e-01 | 6.6e-01 | 5.1e-02 | 5.0e-02 | 6.6e-02 | 3.6e-02 | 1.0e-01 | 2.8e+00 | 2.8e+00 | 7.8e-03 | 7.4e-02 |  |  |  |  |  |  |  |  |  |  |  |  |
| HC (not methane) | 4.7e-02 | 2.1e-01 | 2.5e-01 | 2.6e-03 | 4.9e-02 | 2.7e-02 | 1.5e-02 | 4.3e-02 | 1.1e-01 | 1.4e-01 | 1.8e-01 | 2.1e-01 |  |  |  |  |  |  |  |  |  |  |  |  |
| Sulphur Oxides | 6.4e-02 | 2.9e-01 | 3.6e-01 | 5.7e-03 | 7.0e-02 | 5.9e-02 | 6.1e-02 | 1.2e-01 | 1.9e-02 | 7.8e-02 | -8.9e-02 | -3.0e-02 |  |  |  |  |  |  |  |  |  |  |  |  |
| Carbon Monoxide | 1.3e-01 | 4.9e-01 | 6.2e-01 | 5.8e-03 | 1.4e-01 | 5.5e-02 | 3.1e-02 | 8.6e-02 | 6.5e-02 | 1.2e-01 | 3.3e-02 | 8.8e-02 |  |  |  |  |  |  |  |  |  |  |  |  |
| Carbon Dioxide | 1.4e+01 | 1.1e+02 | 1.3e+02 | 8.4e+01 | 9.7e+01 | 9.6e+00 | 9.4e+01 | 1.0e+02 | 1.3e+02 | 1.4e+02 | 7.1e+01 | 8.1+01 |  |  |  |  |  |  |  |  |  |  |  |  |
| Aldehydes | 1.6e-04 | 1.1e-02 | 1.1e-02 | 5.4e-05 | 2.1e-03 | 1.2e-03 | 1.3e-04 | 1.3e-03 | 1.5e-03 | 2.7e-03 | 1.1e-03 | 2.3e-03 |  |  |  |  |  |  |  |  |  |  |  |  |
| Other Organics | 2.1e-02 | 2.1e-01 | 2.3e-01 | 1.1e-03 | 2.2e-02 | 2.3e-02 | 2.5e-03 | 2.6e-02 | 3.0e-02 | 5.4e-02 | 2.2e-02 | 4.5e-02 |  |  |  |  |  |  |  |  |  |  |  |  |
| Ammonia | 4.7e-06 | 6.9e-05 | 7.4e-05 | 3.9e-07 | 5.1e-05 | 8.0e-06 | 1.3e-06 | 9.3e-06 | 1.0e-05 | 1.8e-05 | 6.5e-06 | 1.5e-05 |  |  |  |  |  |  |  |  |  |  |  |  |
| Lead | 3.5e-06 | 2.0e-08 | 3.5e-06 | 1.1e-08 | 3.5e-06 | 2.3e-09 | 3.5e-10 | 2.6e-09 | 2.9e-09 | 5.1e-09 | 1.8e-09 | 4.1e-09 |  |  |  |  |  |  |  |  |  |  |  |  |
| Methane | 2.5e-04 | 1.5e+01 | 1.5e+01 | 2.9e-05 | 2.8e-04 | 1.5e-04 | 1.3e-04 | 2.8e-04 | 2.2e-04 | 3.7e-04 | 5.0e+00 | 5.0e+00 |  |  |  |  |  |  |  |  |  |  |  |  |
| Kerosene | 1.0e-06 | 2.5e-07 | 1.3e-06 | 1.3e-07 | 1.2e-06 | 1.1e-06 | 1.9e-06 | 3.1e-06 | 7.4e-08 | 1.2e-06 | -4.1e-06 | -2.9e-06 |  |  |  |  |  |  |  |  |  |  |  |  |
| HCI | 1.4e-07 | 2.1e-05 | 2.3e-06 | 1.2e-08 | 1.5e-07 | 2.5e-07 | 3.8e-08 | 2.8e-07 | 3.1e-07 | 5.6e-07 | 2.0e-07 | 4.5e-07 |  |  |  |  |  |  |  |  |  |  |  |  |
| Water Vapour fw | 0.0e+00 | 0.0e+00 | 0.0e+00 | 2.4e+01 | 2.4e+01 | 0.0e+00 | 1.6e+02 | 1.6e+02 | 2.0e+02 | 2.0e+02 | 2.4e+01 | 2.4e+01 |  |  |  |  |  |  |  |  |  |  |  |  |
| Total Air Emissions | 1.4e+01 | 1.3e+02 | 1.4e+02 | 1.1e+02 | 1.2e+02 | 9.9e+00 | 2.6e+02 | 2.7e+02 | 3.4e+02 | 3.5e+02 | 1.0e+02 | 1.1e+02 |  |  |  |  |  |  |  |  |  |  |  |  |
| SW / CW | 1.6e+00 | 4.7e+02 | 4.8e+02 | 2.8e+00 | 4.4e+00 | 9.7e-01 | 1.7e+00 | 2.7e+00 | 3.4e-01 | 1.3e+00 | 5.0e+00 | 6.0e+00 |  |  |  |  |  |  |  |  |  |  |  |  |
| Other | 0.0e+00 | 3.1e+02 | 3.1e+02 | 3.4e+02 | 3.4e+02 | 0.0e+00 | 3.9e+01 | 3.9e+01 | 3.3e+00 | 3.3e+00 | 2.5e+01 | 2.5e+01 |  |  |  |  |  |  |  |  |  |  |  |  |
| Water / Waterborne Wastes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Water | 2.6e+02 | 3.6e+03 | 3.8e+03 | 2.1e+03 | 2.3e+03 | 3.9e+01 | 2.1e+01 | 5.9e+01 | 2.9e+01 | 6.7e+01 | 1.9e+02 | 2.3e+02 |  |  |  |  |  |  |  |  |  |  |  |  |
| Acid | 1.0e-09 | 6.6e-02 | 6.6e-02 | 6.6e-02 | 6.6e-02 | 1.7e-09 | 2.7e-10 | 2.0e-09 | 2.2e-09 | 4.0e-09 | 1.4e-09 | 3.2e-09 |  |  |  |  |  |  |  |  |  |  |  |  |
| Metal Ion | 2.1e-05 | 3.2e-04 | 3.4e-04 | 1.8e-06 | 2.3e-05 | 3.7e-05 | 5.8e-06 | 4.3e-05 | 4.7e-05 | 8.4e-05 | 3.0e-05 | 6.7e-05 |  |  |  |  |  |  |  |  |  |  |  |  |
| Dissolved Solids | 1.3e-02 | 1.8e-01 | 1.9e-01 | 2.5e+00 | 2.5e+00 | 2.1e-02 | 3.4e-03 | 2.4e-02 | 2.7e-02 | 4.8e-02 | 1.8e-02 | 3.9e-02 |  |  |  |  |  |  |  |  |  |  |  |  |
| Suspended Solids | 7.1e-03 | 1.2e-01 | 1.2e-01 | 2.0e-01 | 2.1e-01 | 1.2e-03 | 1.3e-03 | 2.4e-03 | 3.8e-04 | 1.5e-03 | 2.5e-02 | 2.7e-02 |  |  |  |  |  |  |  |  |  |  |  |  |
| BOD | 1.1e-03 | 3.5e-03 | 4.6e-03 | 1.3e-04 | 1.2e-03 | 4.5e-03 | 3.9e-04 | 4.9e-03 | 3.2e-05 | 4.5e-03 | 1.3e-02 | 1.8e-02 |  |  |  |  |  |  |  |  |  |  |  |  |
| COD | 4.0e-03 | 8.7e-04 | 4.8e-03 | 1.3e-05 | 4.0e-03 | 1.3e-04 | 1.1e-04 | 2.4e-04 | 1.3e-04 | 2.6e-04 | 2.5e-02 | 2.5e-02 |  |  |  |  |  |  |  |  |  |  |  |  |
| Phenol | 7.0e-08 | 1.0e-06 | 1.1e-06 | 5.8e-09 | 7.6e-08 | 1.2e-07 | 1.9e-08 | 1.4e-07 | 1.5e-07 | 2.7e-07 | 9.7e-08 | 2.2e-07 |  |  |  |  |  |  |  |  |  |  |  |  |
| Oil | 1.4e-03 | 2.5e-03 | 3.9e-03 | 3.3e-05 | 1.5e-03 | 3.1e-04 | 1.8e-04 | 4.8e-04 | 4.6e-04 | 7.7e-04 | 4.2e-04 | 7.3e-04 |  |  |  |  |  |  |  |  |  |  |  |  |
| Sulphuric Acid | 2.4e-03 | 5.5e-04 | 3.0e-03 | 3.1e-04 | 2.7e-03 | 2.6e-03 | 4.6e-03 | 7.2e-03 | 1.7e-04 | 2.8e-03 | -9.5e-03 | -6.9e-03 |  |  |  |  |  |  |  |  |  |  |  |  |
| Iron | 6.1e-04 | 1.4e-04 | 7.5e-04 | 7.6e-05 | 6.8e-04 | 6.6e-04 | 1.1e-03 | 1.8e-03 | 4.3e-05 | 7.0e-04 | 1.0e-02 | 1.1e-02 |  |  |  |  |  |  |  |  |  |  |  |  |
| Ammonia + NO3 | 1.7e-06 | 2.5e-05 | 2.7e-05 | 1.4e-07 | 1.8e-06 | 2.9e-06 | 4.5e-07 | 3.3e-06 | 3.7e-06 | 6.5e-06 | 2.5e-03 | 2.5e-03 |  |  |  |  |  |  |  |  |  |  |  |  |
| Chromium | 4.1e-09 | 6.0e-08 | 6.5e-08 | 3.4e-10 | 4.4e-09 | 7.0e-09 | 1.1e-09 | 8.1e-09 | 8.9e-09 | 1.6e-08 | 5.7e-09 | 1.3e-08 |  |  |  |  |  |  |  |  |  |  |  |  |
| Lead | 1.8e-09 | 2.7e-08 | 2.8e-08 | 1.5e-10 | 2.0e-09 | 3.1e-09 | 4.8e-10 | 3.6e-09 | 3.9e-09 | 7.0e-09 | 2.5e-09 | 5.6e-09 |  |  |  |  |  |  |  |  |  |  |  |  |
| Zinc | 2.7e-08 | 3.9e-07 | 4.2e-07 | 2.2e-09 | 2.9e-08 | 4.5e-08 | 7.1e-09 | 5.2e-08 | 5.7e-08 | 1.0e-07 | 3.7e-08 | 8.2e-08 |  |  |  |  |  |  |  |  |  |  |  |  |
| Total Water Waste | 3.0e-02 | 1.3e+01 | 1.3e+01 | 2.8e+00 | 2.8e+00 | 3.0e-02 | 1.1e-02 | 4.1e-02 | 2.8e-02 | 5.9e-02 | 8.5e-02 | 1.2e-01 |  |  |  |  |  |  |  |  |  |  |  |  |
| TOTAL | 2.8e+02 | 4.5e+03 | 4.8e+03 | 2.5e+03 | 2.8e+03 | 4.9e+01 | 3.2e+02 | 3.7e+02 | 3.7e+02 | 4.2e+02 | 3.2e+02 | 3.7e+02 |  |  |  |  |  |  |  |  |  |  |  |  |